# Custom Hardware Design

## Introduction

The DEEP3 project’s aim is to investigate hardware/ software co-design methods. It is usual in such methods for the developers to design custom hardware either as a means of interfacing the system to some external function, or as a means of enhancing system performance. This section describes the design of the custom hardware to inter-connect audio speakers to the re-configurable platform for direct audio playback.

## High-Level Design

Once the sound has been decrypted and decoded to a series of PCM samples, it has to be converted from this digital form to an analogue signal that can be interpreted by the speakers. Passive or amplified speakers may be used during playback. Passive speakers are typically used in combination with an amplifier else they produce minimal volume. Amplified speakers have an internal amplifier, usually integrated into one or more of the speakers. The MPEG Audio Layer 1 standard supports stereo, dual-monophonic and mono sound. Mono only requires a single speaker while stereo and dual-monophonic sound each requires two speakers, one for each encoded channel. In order to support all modes of operation there need to be at least two speakers. The final component of the system is a power supply. The speakers may have their own supply direct from the mains. The rest of the system will have to be powered directly from the FPGA. Figure 1 shows a high-level model of the audio interfacing system.

Digital to Analogue Conversion

Amplifier

(Optional)

Analogue Signal

Left Channel

Right Channel

Digital Audio signal from decoder

Power Supply

Figure 1: High Level Model of Playback System

## Digital to Audio Conversion

The simplest method for converting of the digital samples to an analogue signal is to use a digital to analogue converter (DAC) chip. A quick check for Audio DAC chips at a local electronics supplier (Farnell []) revealed two dual audio DACs (DAC chips with support for two input and two output channels on a single chip). Both devices are manufactured by Analog Devices®, each having dual serial input, dual serial output, and a single +5 V Supply. Both operate at 8 x over-sampling frequency and are packaged as 16-pin plastic DIP or SOIC.

Digital to analogue over-sampling is a process whereby and additional values are “interpolated” between the two existing samples. N x over-sampling playback system inserts (N-1) additional values are inserted between the two original samples [3]. Over-sampling is used to remove artefacts.  Another benefit of over-sampling is the reduction of quantisation noise. The interpolation might be performed by a special algorithm, such as the Smith-Gossett [4] algorithm used in some Digital Signal Processors, or be just simple linear, cubic or some n-order polynomial interpolative algorithm.

A dual audio DAC is preferred over two single DACs, as the cost of two single DACs is usually greater than the cost of a single dual audio DAC. Another reason for the choice was the fact that the dual DAC is a complete system on a chip. Systems on a single integrated circuit (IC) tend to consume less power than systems on multiple chips, as there are fewer line drivers. Systems on a chip also tend to be more reliable as there are fewer solder points that could crack during handling and general use.

The major difference between the two dual DACs, sold at Farnell, is the number of bits in a sample. The AD1866 is a 16-bit DAC while the AD1868 is an 18-bit DAC. As only 16-bit resolution is required to meet the minimum criteria for CD quality sound, the AD1866 was selected. Another reason for selecting this chip was that the MP3 encoder/ decoder that the system is based on only supports 16-bit output. “A versatile digital interface allows the AD1866 to be directly connected to all digital filter chips. Fast CMOS logic elements allow for an input clock rate of up to 16 MHz. This allows for operation at 2x, 4x, 8x, or 16x the sampling frequency [of 44.1 kHz] for each channel”[]. The maximum clock rate of the AD1866 is specified to be at least 13.5 MHz.

There are two points where the DAC might be connected to the FPGA board, the 50-pin Aux I/O header and one of the two PMC interfaces. The 50-pin unassigned I/O header is the more suitable as it is designed for custom interfaces unlike the PMC points which are really for data conforming to the PCI standard. Table 1 describes each of the AD1866’s 16-pins. Figure 2 shows how the DAC might be connected.

| Pin # | Pin Name | Description |
| --- | --- | --- |
| 1 | VL | Digital Supply (+5V) |
| 2 | LL | Left Channel Latch Enable Pin |
| 3 | DL | Left Channel Data Input Pin |
| 4 | CLK | Clock Input Pin |
| 5 | DR | Right Channel Data Input Pin |
| 6 | LR | Right Channel Latch Enable Pin |
| 7 | DGND | Digital Common Pin |
| 8 | VBR | Right Channel Bias Pin |
| 9 | VS | Analogue Supply (+5 V) |
| 10 | VOR | Right Channel Output Pin |
| 11 | NRR | Right Channel Noise Reduction Pin |
| 12 | AGND | Analogue Common Pin |
| 13 | NRL | Left Channel Noise Reduction Pin |
| 14 | VOL | Left Channel Output Pin |
| 15 | VS | Analogue Supply (+5 V) |
| 16 | VBL | Left Channel Bias Pin |

Table 1: Pin Descriptions for AD1866 (After [Error: Reference source not found])

Figure 2: Interfacing the AD1866 to the 50-pin AUX (After [Error: Reference source not found])

The circuit is essentially the DAC channel outputs connected to RC low-pass (anti-alias) filters to filter out the frequencies above upper frequency limit of humans (20 kHz). It is usual to filter the output before amplification or have an integrated filter/ amplifier after the DAC stage.

Figure 3 shows how the various control signals must be operated in order to produce an output signal on either output channel. The input data bits on data left (DL) and data right (DR) are serially clocked into input registers on the rising edge of CLK. The respective DAC outputs are updated by the falling edges of LL and LR after 16-bits of data has been clocked into the serial registers. The data is transmitted to the AD1866 in a bit-stream composed of 16-bit words in twos complement form with the most significant bit first. The output voltage produced is 1 V at load currents up to 1 mA. VBL and VBR both provide a dc reference voltage equal to the centre of the output voltage swing (about +2.5V).

Figure 3: AD1866 Control Signals (After [Error: Reference source not found])

According the datasheet for the DAC [Error: Reference source not found], the minimum voltage that will be recognised as a logic high level on one of the input pins is 2.4V. The maximum voltage for a digital low on one of these inputs is 0.8V. The power supply pins VL and VS are both specified to work between 4.75V and 5.25V, though 5V is typical. These supplies may operate at voltages as low as 3.5V. The power supply must therefore supply a voltage of at least 4.75V in order to meet the specified voltage needed by the DAC. How LL, DL, CLK, DR, LR, DGND and all +5V and ground supplies are connected to the FPGA will be discussed in the power supply design section. The channel outputs could either be connected to a single 3-pole jack socket, separately to two 2-pole jack sockets, or to the speakers directly.

The frequency of the input clock is given by the equation:

Frequency = sampling frequency * number of bits per sample * over-sampling rate

The sampling frequency is the value in the MP3 header.

Amplifier and Speakers

The amplifier converts the analogue signal from the DAC to the chosen user sound “loudness” value, which must be between the positive and negative supply voltage rails of the speakers. Two 5-10 Watt (W) Root Mean Squared (RMS) passive speakers will produce a discrete sound without amplification. Two 20-50 W RMS active speakers will produce a very loud sound. As stated previously the amplifier may be part of the speaker unit(s) or may be a stand-alone unit. We have chosen active/amplified speakers for several reasons. First, it simplifies the custom hardware design by having a prefabricated speaker and amplifier unit. This also reduces risks and has the advantage of producing a higher quality product than might be possible if both the speakers and amplifiers are built from components of the shelf. Finally, it enables greater reuse. The same speakers that are used to play sound via the sound card in the initial project stages can be reused in this later stage leading to lower development costs.

## Power Supply

Ultimately, the filter and the digital to analogue converter must be connected to the FPGA in order to convert the samples to audio output. The RC1000-PP from Celoxica supports many input/output standards. The target architecture combines the RC1000-PP with a Xilinx® Virtex™ XCV1000 FPGA chip. This chip only supports a subset of the I/O standards. Table 2 shows the standards supported by the XCV1000.

| I/O Standard ([], []) | Performance Target | Typical Application |
| --- | --- | --- |
| LVTTL (3.3V) | 250 MHz | General Purpose |
| LVCMOS (2.5V) | 250 MHz | General Purpose |
| PCI 33MHz (3.3V, 5V) | 33 MHz | PC & Embedded |
| PCI 66MHz (3.3V) | 66 MHz | PC & Embedded |
| GTL (1.5V, 2.5V, 3.3V) | 100 MHz | Backplane |
| GTL+ (1.5V, 2.5V, 3.3V) | 200 MHz | Backplane |
| HSTL Class I (1.5V) | 250 MHz | Memory & Switch Fabric |
| HSTL Class III (1.5V) | 250 MHz | Memory & Switch Fabric |
| HSTL Class IV (1.5V) | 250 MHz | Memory & Switch Fabric |
| SSTL2 Class I & II (2.5V) | 200 MHz | DDR 1 SDRAM |
| SSTL3 Class I & II (3.3V) | 166 MHz | SDRAM |
| CTT (3.3V) | 250 MHz | Backplane |
| AGP 1x (3.3V) | 66 MHz | 3-D Graphics |
| AGP 2x (3.3V) | 133 MHz | 3-D Graphics |

Table 2: I/O Standards Supported (After [])

| Acronym | Long Form |
| --- | --- |
| LVTTL | Low Voltage TTL |
| LVCMOS | Low Voltage CMOS |
| PCI | Peripheral Component Interconnect |
| GTL | Gunning Transceiver Logic Terminated |
| GTL+ | Gunning Transceiver Logic Plus |
| HSTL | High-speed Transceiver Logic |
| SSTL2 | Stub Series Terminated Logic for 2.5V |
| SSTL3 | Stub Series Terminated Logic for 3.3V |
| CTT | Centre Tap Terminated |
| AGP(1x, 2x) | Advanced Graphics Port |

Table 3: I/O Standards Acronyms Expanded

For input signals into the clock pin and data input pins of the DAC, the FPGA I/O standard must support switching at speeds up 13.5MHz with a voltage greater than 2.4V. The positive power supply pins require an I/O standard that can drive a CMOS or TTL input at a voltage to at least 4.75V.

All of the 3.3V and 5.0V standards listed in Table 2 and expanded in Table 3 are appropriate for the input signals to the DAC both in terms of voltage level and switching frequency. The 2.5V standards could be used but they allow only a small deviation in the signal level and so are not highly favoured. The I/O standard is being applied to an embedded general-purpose design and so this further narrows the possible I/O standards to only one possibility, LVTTL. The DAC input pins will therefore be all connected directly to the FPGA via a LVTTL interface. This is possible because the digital input pins of the AD1866 are TTL compatible.

Only PCI meets the criteria for at least 4.75V at the positive power supply pins of the DAC but LVTTL at 3.3V must be used because of a design decision based on the typical application uses of the various I/O standards. It was therefore necessary to design an interface to convert these low voltages to over 4.75V DC. A DC-DC converter was chosen for this interface.

The part used had to support inputs from 2.5V to 3.3V. It also had to be low cost. The MAX619 [] step-up charge-pump DC-DC converter from MAXIM is such a chip. It converts voltages from 2.0V to 3.6V to an output between 4.8V and 5.2V with minimal interfacing components (only four external capacitors). Not only is the chip inexpensive (£2.32 from Farnell), the extra components required are also very low cost. The chip also features a shutdown pin (SHDN) that places the device in a low-power shutdown mode and zeros the output voltage, effectively turning off any connected circuitry. The output current supply is more than adequate for the application. Figure 4 shows a schematic of the power supply interface. The input signal on the IN pin is direct from the FPGA and so is the SHDN pin’s signal. The pins requiring 5.0V in the filter and DAC are connected to the OUT pin of the MAX619. The negative power supply rail of all chips is direct from the FPGA using a LVTTL 3.3V interface.

Figure 4: Power Supply Design

## Conclusion

In this section a high level and a detailed schematic design level of the custom audio interface were described. The power supply to the filter and the digital to analogue converter are from the FPGA via a DC-DC step-up converter. The clock signal, left and right channel data, and left and right channel enable signals of the DAC are direct from LVTTL 3.3V configured pins of the 50 pin AUX header of the FPGA. Some processing is required before data is sent to either channel of the DAC. This is essentially an interpolation algorithm that has yet to be specified. The system reuses the speakers from earlier stages in the project’s lifecycle in order to reduce cost and simplify design. These speakers have an internal amplifier for volume. The speakers connect to the filter via a single 3-pole audio socket. Table 4 shows the major components used in the design and the cost of each of these components.

| Part Description | Manufacturer | Part Code | Supplier | Supplier Code | Price (£) |
| --- | --- | --- | --- | --- | --- |
| Dual Op Amp | EBV ELEKTRONIK | LM358D | Farnell | 300-3681 | 0.27 |
| DC-DC converter | MAXIM | MAX619 | Farnell | 702-614 | 2.32 |
| Dual Audio DAC | Analog Devices | AD1866 | Farnell | 595-020 | 19.03 |

Table 4 : Major Parts List

# Custom Hardware Test Plan

## Introduction

System designs always need to be adequately tested for functional correctness and adherence to non-functional requirements. In this section, the method by which the custom hardware will be tested is described.

## Tests

The design needs to be tested for its frequency response, amplitude step response, for the correct signals on each input and output pin, and for short circuits. Frequency response tests verify the implementation of the audio filters. The step response is to prove that for all possible serially inputted voltage signal into the DAC, the analogue output is of the correct signal level before and after filtering, within expected error bands. Other tests are there to detect spurious signals that may cause unexpected results in the other tests. The tests will be conducted in the following order: (1) Test for short circuits, (2) Tests for correct signal levels, (3) frequency response and (4) step response.

### Test for short circuits

Shorts can be detected first by physical inspection of each track and solder joint. Once all obvious shorts have been identified using the first method, they should be resolved. Next, using a multi-meter with a short circuit detector, each solder point should be tested for short circuits. Any faulty points found should be remedied.

### Tests for correct signal levels

A voltmeter will be used to confirm that each signal pin is at the expected voltage.

### Frequency Response Tests

As it would be too time consuming to test the response of the system to every frequency in the human auditory range, a subset of frequencies must be studied. It has been decided that frequencies should be tested on a logarithmic scale. The following output frequencies will be tested: 20 Hz, 30Hz, 40Hz, 50Hz, 60Hz, 70Hz, 80Hz, 90Hz, 100Hz, 200Hz, 300Hz, 400Hz, 500Hz, 600Hz, 700Hz, 800Hz, 900Hz, 1000Hz, 2.0kHz, 3.0kHz, 4.0kHz, 5.0kHz, 6.0kHz, 7.0kHz, 8.0kHz, 9.0kHz, 10.0kHz, 20.0kHz and 30.0kHz.

The simplest way to verify the frequencies is to generate square waves at the frequency under investigation on both audio channels. The frequency of the clock signal into the DAC should be the over-sampling rate times the frequency of the signal at the output on either channel before filtering. If there is any difference between these two signals first check that the clock signal is correct for the test involved. If it is correct, re-check the signal levels on the various DAC I/O pins. The square waves should all have a peak to peak amplitude 2.0V (full range of values supported by DAC output). The input signals to produce the required output may be generated by a logic analyser or a test program executing on the FPGA. An oscilloscope may be used to analyse the output signals.

All frequencies after filtering should be the same as the value before the filter except for the signal at 30.0kHz.

### Amplitude Response Tests

Like the frequency response tests, the amplitude response tests cannot be exhaustive. Twenty-five equally spaced amplitudes spanning the full range of output voltage levels of the DAC will be considered. Each amplitude will be tested with a sine wave output signal at a frequency of 1kHz. Again the input signals to produce the required output may be generated by a logic analyser or a test program executing on the FPGA. The output signals may be analysed using an oscilloscope.

## Conclusion

In conclusion the custom hardware will be validated and verified by a number of means including physical inspection and testing. The number of possible input frequencies and amplitudes prevents the system from being exhaustively tested in the allocated time. The system will be tested using a subset of these frequencies and amplitudes.

# Appendix:

[?] Farnell Online, http://www.farnell.com

[?] Analog Devices, Single Supply Dual 16-bit Audio DAC, AD1866 Data Sheet, Rev. 0.

[?] Celoxica, Handel-C Language Reference Manual, Version 2.1

[?] Xilinx®, Virtex™ 2.5V Field Programmable Gate Arrays Data Sheet, v2.5, April 2, 2001

[?] Altera, Stratix Differential I/O standards, http://www.altera.com/products/devices/stratix/features/stx-differential_io.html

[?] MAXIM Regulated 5V Charge-Pump DC-DC Converter Data Sheet (MAX619), MAXIM, Rev 2, 5/96