# An Overview MPEG-1 Audio Layer III

In this section we provide a high level description of the functions needed to convert an audio sample into a MPEG-1 Audio Layer III stream, and how to convert this stream back into audio and play it. First a short introduction is given to establish the role of this standard and the concept on which the standard is based. Next some features of MPEG/ Audio are considered, followed by a description of the audio encoder and decoder, and finally the method for playing the compressed sound is explained.

## Introduction

The Moving Pictures Experts Group (MPEG) Audio Layer-3 [ISO/ IEC 11172-3:1993] (MPEG-1 Layer-3 or MP3) “has generated phenomenal interest among Internet users, or at least among those who want to download highly-compressed digital audio files at near-CD quality.”Error: Reference source not found CD quality is defined as stereophonic 16-bit digital (Pulse Coded Modulation) sound sampled at 44.1 kHz.

Since its inception, MP3 has emerged as the main tool for Internet audio delivery because of a number of reasons. The first reason is that MPEG is an open (i.e. non-proprietary) standard. “While there are a number of patents covering MPEG Audio encoding and decoding, all patent-holders have declared that they will license the patents on fair and reasonable terms to everybody.”Error: Reference source not found Details about licensing can be found at http://www.mp3licensing.com/. To ensure interoperability, the standard makers made example implementations of the standard public to help implementers understand the potentially ambiguous natural language text of the specification.

The proliferation of MP3 is also as a result of the improved computer technology at lower costs. Once computer performance increased to the point where the audio decoders and even encoders could execute in real-time, sound cards became a standard peripheral sold with almost every computer, the speed of Internet access increased, and CD-ROM and CD-Audio writers increased in availability, there seemed to nothing holding the standard back from widespread acceptance. “In short, MPEG Layer-3 had the luck to be the right technology available at the right time.”Error: Reference source not found

“The MPEG/audio compression algorithm is the first international standard for digital compression of high-fidelity audio. Other audio compression algorithms address speech only applications [for example Code Excited Linear Prediction (CELP)] or provide only medium-fidelity audio compression performance [for example Adaptive Differential Pulse Code Modulation (ADPCM)].”

MPEG/audio achieves its great feats of compression while maintaining high sound quality by using a model of how the human mind perceives sound (psychoacoustics). “The MPEG/audio algorithm compresses the audio data in large part by removing the acoustically irrelevant parts of the audio signal. That is, it takes advantage of the human auditory system's inability to hear quantisation noise under conditions of auditory masking. This masking is a perceptual property of the human auditory system that occurs whenever the presence of a strong audio signal makes a temporal or spectral neighbourhood of weaker audio signals imperceptible.... MPEG/audio works by dividing the audio signal into frequency sub-bands that approximate critical bands, then quantising each sub-band according to the audibility of quantisation noise within that band. For the most efficient compression, each band should be quantised with no more levels than necessary to make the quantisation noise inaudible.”Error: Reference source not found

MPEG “audio consists of three operating modes called “Layers”, with increasing complexity and performance, named Layer-1, Layer-2 and Layer-3. Layer-3, with the highest complexity, was designed to provide the highest sound quality at low bit-rates (around 128 kbit/s for a typical stereo signal).”

## MPEG Layer-3 Audio Features

The MPEG standard was designed to be flexible, taking into account typical applications of the standard, while leaving enough leeway for atypical applications. In this section we describe a number of the standard’s features. First it is important to state “there is no main file header in an MPEG audio [bit stream; includes file(s)]. [Instead the stream] is built up from a succession of smaller parts called frames. A frame is a data block with its own header and audio information.” Separating the audio bit stream into frames “makes features such as random access, audio fast forwarding, and audio reverse possible.”Error: Reference source not found

The audio frame is the main data structure of MP3. An audio frame consists of one header, an optional CRC, side information, some audio data and optional ancillary data. The header can be further divided into frame sync (SYNC), MPEG Audio version ID, layer description, protection bit, bit rate index, sampling rate frequency index, padding bit, private bit, audio channel mode, mode extension, copyright bit, original bit (is the data original work or a copy of original media), and emphasis (should the decoder “re-equalize” the sound?).   Figure 1 below shows the frame header represented visually.

Figure 1: The MP3 frame header represented visually’

Another model of the structure of an audio frame is shown below in Figure 2.

Header (32)

CRC (0, 16)

Side Information (136, 256)

Main Data: not necessarily linked to this frame

Ancillary Data

Figure 2: Frame format for Layer 3Error: Reference source not found

In this second model, the number of bits for each section is shown in parenthesis. The CRC and the ancillary data are optional. In particular the ancillary data section is user defined and might be used to transmit information about the song such as the artist or the title (usually as an ID3 tag). The side information is 136 bits in single channel mode and 256 bits in dual channel mode. The usage of the side information and the CRC are explained below as well as other features of Layer 3.

### Bit Reservoir

MP3 processes the audio data in frames of 1152 sound samples. The use of a variable bit rate means that the coded data does not fit into a fixed length frame. “The encoder can [therefore] donate bits to a reservoir when it needs less than the average number of bits to code a frame. Later, when the encoder needs more than the average number of bits to code a frame, it can borrow bits from the reservoir.”Error: Reference source not found The “borrowing” just means that the system notes that there is more space left over from a previous frame where a future frame may write data. These borrowed bits can only come from past frames. As a result of this borrowing, the start of a frame does not necessarily follow its header so “the Layer 3 bit-stream includes a 9-bit pointer, “main_data_begin”, with each frame’s side information that points to the location of the starting byte of the audio data for that frame.”Error: Reference source not found The side information section contains other information necessary for decoding the main data. Figure 3 shows the use of the bit reservoir to obtain a near constant frame size despite the variable bit rate.

Figure 3: Layer 3 bit-stream diagramError: Reference source not found

“Although the main_data_begin limits the maximum variation of the audio data to 29 bytes (header and side information are not counted because for a given mode they are of fixed length and occur at regular intervals in the bit stream), the actual maximum allowed variation will often be much less. For practical considerations, the standard stipulates that this variation cannot exceed what would be possible for an encoder with a code buffer limited to 7,680 bits. Because compression to 320 kbits/sec with an audio sampling rate of 48 kHz requires an average number of code bits per frame of

1152 (samples/frame) * 320,000 (bits/sec) / 48,000 (samples/sec) = 7680 bits/frame

“absolutely no variation is allowed for this coding mode.”Error: Reference source not found

### Audio Channel Modes

“The compressed bit-stream can support one or two audio channels in one of 4 possible modes:

- A monophonic mode for a single audio channel,

- A dual-monophonic mode for two independent audio channels (this is functionally identical to the stereo mode [and could be used to encoded different language versions of the audio]),

- A stereo mode for stereo channels with a sharing of bits between the channels, but no joint-stereo coding, and

- A joint-stereo mode that either takes advantage of the correlations between the stereo channels or the irrelevancy of the phase difference between channels, or both.”Error: Reference source not found By taking these correlations into account, one can achieve more efficient combined coding of the left and right channels of a stereophonic audio signal.Error: Reference source not found

### Sampling frequency

MP3 works at different sampling frequencies, which are dependent on the version of MP3 in use and determine quality of sound on playback, and affects the amount of data stored. “MPEG-1 defines audio compression at 32 kHz, 44.1 kHz and 48 kHz. MPEG-2 extends [MPEG-1 by adding half rates of the former sampling rates]… i.e. 16 kHz, 22.05 and 24 kHz.”Error: Reference source not found

### Bit-rate

MPEG Audio Layer-3 supports both fixed and variable bit rates. There are standard rates in the range 8 kbps to 320 kbps as well as an option for “a “free” bit rate mode to support fixed bit rates other than the predefined”Error: Reference source not found ones.  Allocating different bit rates for any two frames within the bit-stream will result in a variable bit rate (VBR), while a constant bit rate (CBR), is specified by having the same bit rate for each frame in the stream. “Layer-3 decoders must support switching of bit rates from audio frame to audio frame.”Error: Reference source not found

The difference between CBR and VBR are a little subtler than just having different bit rates for frames in the stream. CBR gives the system greater control over the size of each frame, sometimes to the detriment of sound quality. The encoder attempts to keep each frame a fixed size. “If an encoder does not find a way to encode a block of music data with the required fidelity within the limits of the available bit-rate, it “runs out of bits”. This may lead to the deletion of some frequency lines, typically affecting the high-frequency content.”Error: Reference source not found

VBR encoding is a method that ensures high audio quality by allocating an appropriate (minimum) number of bits per second, depending on the complexity of the audio. A silence would be allocated a lower bit-rate while a complex portion of the sound would be allocated a higher rate with less compression and higher sound fidelity. As a result, the average bit-rate of the VBR stream may be lower than that of the constant bit rate. When using VBR, one must specify a quality factor that somehow relates (implementation dependent) to the range of bit-rates used during encoding.

As a rule of thumb, many writers suggest one that should use VBR encoding when consistent audio quality is the top priority and CBR if there is there are storage limitations and VBR produces too large a file. If one requires predictable behaviour (e.g. in a real-time environment), or there is limited bandwidth on the medium through which the data is to stream, then CBR should be chosen.

### Cyclic Redundancy Check (CRC)

Audio frames may have an optional 16-bit CRC checksum. If used, the checksum comes after the frame header but before the audio data. Calculating the CRC for the received audio data and comparing it with the received CRC may identify errors in the received bit stream. If a frame is corrupted, the normal procedure is either to repeat the previous frame or mute the corrupted frame [ISO/ IEC 11172-3, 1993]. Only bits 16 to 31 in the header and the side information need to be used to generate the CRC word. This is because if there is an error in any of these values the whole frame could be misinterpreted, whereas an error in the audio data might only manifest itself as an audible glitch.

### Others

The other parts of the frame header are informative. They provide extra information about the stream but are not very important to the decoding process (though they are needed, along with the others, to allow the decoder to identify the start of a frame). For more details on the structure of MP3 header frames, side information and CRC calculation, including the actual lookup tables necessary to derive certain details from the bit settings, go to http://www.mp3-tech.org/programmer/frame_header.html or http://www.iso.ch or see the Appendix.

## MP3 Encoding Algorithm

“MPEG/audio is a generic audio compression standard. Unlike vocal-tract-model coders specially tuned for speech signals, the MPEG/audio coder gets its compression without making assumptions about the nature of the audio source.”Error: Reference source not found Figure 4 shows the block diagram of a typical MP3 encoder.

Figure 4: Typical MP3 Encoder Error: Reference source not found

As the diagram shows, the input audio stream enters the system at two blocks simultaneously. One of these blocks is an analysis filter-bank. The other is called the perceptual model (or psycho-acoustic model).  The minimum equipment needed to do MP3 encoding is an Intel® Pentium® 133 MHz processor (without MMX support) and 32 MB of Random Access Memory (RAM)Error: Reference source not found. The sections below describe the basic function of each of the blocks.

### Filter bank

MP3 uses a hybrid filter-bank built by cascading a poly-phase filter-bank and a Modified Discrete Cosine Transform (MDCT) filter-bank. The poly-phase filter-bank makes Layer-3 more similar to Layer-1 and Layer-2. Layer-3 needs to be like the other layers because the standard requires new layers to be able to decode data produced by previous layer versions. The hybrid filter-bank divides the input into a number of finer frequency sub-bands. This increases the potential for redundancy removal, leading to greater compression levels. “As a further positive result of the higher frequency resolution, the error signal can be better controlled, allowing finer tracking of the masking threshold.”Error: Reference source not found

### Perceptual model

“The perceptual model mainly determines the quality of a given encoder implementation…. The perceptual model either uses a separate filter-bank … or combines the calculation of energy values (for the masking calculations) and the main filterbank. The output of the perceptual model consists of values for the masking threshold or the allowed noise for each coder partition. In Layer-3, these coder partitions are roughly equivalent to the critical bands of human hearing. If the quantisation noise can be kept below the masking threshold for each coder partition, then the compression result should be indistinguishable from the original signal.”Error: Reference source not found

### Quantisation and Coding

This block uses the signal-to-mask ratios to decide how to distribute the total number of code bits available in such a way as to limit the number of possible amplitudes for each sub-band signal while minimising the audibility of the quantisation noise. “Quantisation is done via a power-law quantiser…. Larger values are automatically coded with less accuracy, and some noise shaping is already built into the quantisation process. The quantised values are coded by Huffman coding. To adapt the coding process to different local statistics of the music signals, the optimum Huffman table is selected from a number of choices.” Error: Reference source not found The Huffman tables (one for each sub-band) are another major data structure in MPEG-1 Audio Layer III. They allow the data to be compressed but without degrading the quality.

### Bit-stream Encoding

The bit-stream encoder is the last block in the process. It “takes the representation of the quantised subband samples and formats this data and side information into a coded bitstream. Ancillary data not necessarily related to the audio stream can be inserted within the coded bitstream.”Error: Reference source not found The header to the audio frame is attached here, as well as the CRC, if used.

On completion of all these stages it is not unusual to achieve the following compression ratios and sound qualities:

| sound quality | bandwidth | mode | bitrate | reduction ratio |
| --- | --- | --- | --- | --- |
| telephone sound | 2.5 kHz | mono | 8 kbps * | 96:1 |
| better than short-wave | 4.5 kHz | mono | 16 kbps | 48:1 |
| better than AM radio | 7.5 kHz | mono | 32 kbps | 24:1 |
| similar to FM radio | 11 kHz | stereo | 56...64 kbps | 26...24:1 |
| near-CD | 15 kHz | stereo | 96 kbps | 16:1 |
| CD | >15 kHz | stereo | 112..128kbps | 14..12:1 |
| *Fraunhofer uses a non-ISO extension of MPEG Layer-3 for enhanced performance ("MPEG 2.5") |  |  |  |  |

Table 1: Typical performance data for MPEG Layer-3Error: Reference source not found

An audio CD can store a maximum of 74 minutes of sound, requiring 650 MB of storage space. MP3 can, according to Fraunhofer, produce the same quality of sound at about one twelfth the space required for a CD, about 750 KB per minute of sound.

### Factors determining encoder quality

“To date, large-scale and well-controlled listening tests are still the only method available for comparing the performance of different coding algorithms and different encoders…. These tests aim to stress the encoders under worst-case conditions… and then evaluate the performance of the encoders under test”Error: Reference source not found.

As stated above, the quality of the perceptual model is a major determining factor for the perceived encoder quality. Implementing a very simple or no perceptual model can result in a fast encoder, but which performed poorly in some listening tests. There is obviously a trade-off between speed of encoding and sound quality.

“Especially at low bit-rates and low sampling frequencies, there is a mismatch between time resolution of the coder and the time structure of some signals. This effect is most noticeable on speech signals and when listening via headphones. As a single voice tends to sound like it has been recorded twice and then overlaid, this effect is sometimes called “double-speak”.”Error: Reference source not found If sound quality is important then the encoder should support VBR and the audio input into the encoder should be sampled at a suitably high frequency.

## MP3 Decoding Algorithm

A decoder is considered a valid MP3 decoder if the maximum deviation of the decoded signal is within a certain limit from a reference decoder which uses double precision arithmetic accuracy. “This allows us to build decoders running both on floating-point and fixed-point architectures. Depending on the skills of the implementers, fully-compliant high-accuracy Layer-3 decoders can be constructed with down to 20-bit arithmetic word length, without using double-precision calculations.”Error: Reference source not found In general terms, “the decoder deciphers [the encoded] bit stream, restores the quantised sub band values, and reconstructs the audio signal from the sub band values.”Error: Reference source not found Figure 5 shows the decoding process. The minimum equipment needed for these processes is a system with processing power equal to an Intel® Pentium® 75 MHz system with 16 MB of Random Access Memory (RAM).

Figure 5: Basic functionality of MP3 DecoderError: Reference source not found

### Decoding of bit-stream

First the incoming bit stream undergoes synchronisation. Here the contents of the bit-stream are identified and passed to succeeding blocks of the decoder. The process is also called bit-stream unpacking in literatureError: Reference source not found. This is done by finding the header frame and removing pertinent information. If CRC checking is enabled the CRC is extracted and used for error detection. If an error is detected, the frame is discarded and the system searches for a new header. The input bit-stream is also decoded (and decompressed) here using a Huffman table, and expanded to its raw form before the loss-less compression. If included, the ancillary data is also extracted.

### Inverse Quantisation

The decoded bit-stream is decompressed even further by multiplying each sub-band by the inverse of the value by which they were scaled during the quantisation phase of the encoder. On completion the frequency samples are reconstructed with some error related to the quantisation process.

### Synthesis Filter-bank

The synthesis filter-bank performs the inverse operation of the analysis filter-bank, thereby converting the sound data from the frequency domain, to the time domain, using the Inverse Modified Discrete Cosine Transform (IMDCT) followed by the synthesis of the poly-phase filter-bank. The sum effect is the reconstruction of the audio signal. The output of this stage is a digital Pulse Code Modulation (PCM) signal.

## Generating the Output Sound

Once the sound has been decoded it is ready for playback. During playback the digital sound must first be converted to an analogue form, then outputted via speakers. Two types of speakers are generally used to playback MP3 sound, namely passive and amplified speakers. Passive speakers are typically used in combination with an amplifier. They produce minimal volume without amplification. Amplified speakers have their own amplifier, usually integrated into one or more of the speakers. The MP3 standard requires a decoder to be able to support stereo sound. Stereo is defined as “designating sound transmission from two sources through two channels.” This means that during playback there should be at least two speakers, one for each encoded channel. Figure 6 shows the operations occurring during playback.

Digital to Analogue Conversion

Amplifier

(Optional)

Analogue Signal

Left Channel

Right Channel

Digital Audio signal from decoder

Figure 6: Playback of the audio stream

### Digital to Analogue Conversion

This process may be undertaken by the computer’s sound card or by custom hardware. If custom hardware is used a digital to analogue converter (DAC) chip is utilised. A quick check for Audio DAC chips at a local electronics supplier (Farnell) revealed two dual audio DACs (DAC chips with support for two input and two output channels on a single chip). Both devices are manufactured by Analog Devices®, each having dual serial input, dual serial output, and a single +5 V Supply. Both operate at 8 x over-sampling frequency and are packaged as 16-pin plastic DIP or SOIC.

“Digital to analogue over-sampling is a process whereby the player reads two samples, and additional values are “interpolated” between the two. In an 8x over-sampling playback system, 7 additional values are inserted between the actual single samples.” Over-sampling is used to remove artefacts.  Another benefit of over-sampling is the reduction of quantisation noise. The interpolation might use a special algorithm, such as Smith-Gossett algorithm used in some Digital Signal Processors, or be just simple linear interpolation.

A dual audio DAC is preferred, over two single audio DACs, as the cost of two single DACs is usually greater than the cost of a single dual audio DAC. Another reason for the choice was the fact that the dual DAC is a complete system on a chip. Systems on a single integrated circuit (IC) tend to consume less power than systems on multiple chips, as there are fewer line drivers. Systems on a chip also tend to be more reliable as there are fewer solder points that could crack during handling and general use.

The major difference between the two dual DACs, sold at Farnell, is the number of bits in a sample. The AD1866 is a 16-bit DAC while the AD1868 is an 18-bit DAC. As only 16-bit resolution is required to meet the minimum criteria for CD quality sound, the AD1866 was selected. “A versatile digital interface allows the AD1866 to be directly connected to all digital filter chips. Fast CMOS logic elements allow for an input clock rate of up to 16 MHz. This allows for operation at 2x, 4x, 8x, or 16x the sampling frequency [of 44.1 kHz] for each channel.” The maximum clock rate of the AD1866 is specified to be at least 13.5 MHz.

There are two points where the DAC might be connected to the FPGA board, the 50-pin Aux I/O header and one of the two PMC interfaces. The 50-pin unassigned I/O header is the more suitable as it is designed for custom interfaces unlike the PMC points which are really for data conforming to the PCI standard. Figure 7 shows how the DAC might be connected.

Figure 7: Interfacing the AD1866 to the 50-pin AUX

The circuit is essentially the DAC channel outputs connected to RC low-pass (anti-alias) filters to filter out the frequencies above 20 kHz. LL, DL, CLK, DR, LR, DGND and all +5V and ground supplies are from the FPGA. The channel outputs could either be connected to a single 3-pole jack socket, separately to two 2-pole jack sockets, or to the speakers directly.

Table 2 describes each of the AD1866’s 16-pins.

| Pin# | Pin Name | Description |
| --- | --- | --- |
| 1 | VL | Digital Supply (+5V) |
| 2 | LL | Left Channel Latch Enable Pin |
| 3 | DL | Left Channel Data Input Pin |
| 4 | CLK | Clock Input Pin |
| 5 | DR | Right Channel Data Input Pin |
| 6 | LR | Right Channel Latch Enable Pin |
| 7 | DGND | Digital Common Pin |
| 8 | VBR | Right Channel Bias Pin |
| 9 | VS | Analogue Supply (+5 V) |
| 10 | VOR | Right Channel Output Pin |
| 11 | NRR | Right Channel Noise Reduction Pin |
| 12 | AGND | Analogue Common Pin |
| 13 | NRL | Left Channel Noise Reduction Pin |
| 14 | VOL | Left Channel Output Pin |
| 15 | VS | Analogue Supply (+5 V) |
| 16 | VBL | Left Channel Bias Pin |

Table 2: Pin Descriptions for AD1866Error: Reference source not found

The frequency of the input clock is given by the equation

Frequency = sampling frequency * number of bits per sample * over-sampling rate

The sampling frequency is the value in the MP3 header.

### Amplifier

The amplifier converts the analogue signal from the DAC to the chosen user sound “loudness” value, which must be between the positive and negative supply voltage rails of the speakers. Two 5-10 Watt (W) Root Mean Squared (RMS) passive speakers will produce a very discrete sound without amplification. Two 20-50 W RMS active speakers will produce a very loud sound. As stated previously the amplifier may be part of the speaker unit(s) or may be a stand-alone unit.

## Conclusion and Summary

In this section an overview of the basic functionality of a MP3 encoder and a MP3 decoder was presented. The features that make Layer 3 achieve high levels of compression while maintaining high sound quality were also introduced. The two main data structures for successful encoding and decoding, namely the audio frame structure and the Huffman tables, were also identified. Frames allow skipping, fast-forwarding and reversing of the audio stream providing similar functions available to CD players. Finally the method to produce sound during playback was described. The ISO/ IEC 11172-3:1993 standard allows implementations to be either floating or fixed point with a minimum word length of 20-bits for the arithmetic operations. This latter point is important as the hardware implementation language, Handel-C does not provide support for floating point numbers, and a small word length will result in a design that uses less chip area than one with a longer word.

# MP3 Encoder selection:

The following section is intended to describe the MP3 encoder that decided to be used and the criteria for selection of the encoder. A list of available encoder with source code from the Internet is also included.

## The Criteria & Justification:

The adjective “BEST” is the obvious selection criterion for anything where there is a choice involved. The factors that determine this are however, a bit problematic especially for choosing an encoder, since there are different factors that in total contrast and mostly a trade off.  The following criteria were drawn for selecting the codec that will be in used here in.

- Variable Bit Rate encoding

- Fix Point  & Open Source

- Sound quality

- Speed

- Result of Independent comparisons

- Public domain usage frequency.

### Bit-Rate encoding:

Bit rate encoding has a direct effect on quality of the compressed sound and the size of compressed sound file or compression ratio. There are three different bit rate encoding used by different encoders. Constant Bit Rate (CBR), Average Bit Rate (ABR) and Variable Bit Rate (VBR) have direct effect on compression and sound quality [14,17].

CBR as the name suggests is an encoding technique that allocates a constant bit-rate and encodes the stream at that rate. This method undermines the sound quality such that, if the bit-rate is low, and music is complex, it gets distorted. If the bit of music is simple, for example, silence, it introduces a “hiss” sound.

ABR is a technique which allows the user to choose an average bit rate, and the encoder adds bits if necessary.

VBR, on the other hand uses a method that makes intelligent decisions during encoding by allocating as much bandwidth as required. The draw back is that the size of the compressed sound file is unpredictable. Where high sound quality is needed, this method is the best.

### Open Source & Fixed-point:

The system is to be implemented on an FPGA, and as such a fixed-point implementation is preferred.  Libraries are available for implementing floating point operations in Handel-C, however this adds a certain degree of complexity.

Naturally, it is necessary for us to use an open-source implementation of the MP3 algorithm, as we require all the source code for both design methodologies.  Furthermore, we have a limited budget and would prefer not to purchase licensed software.  Open source also allows us to concentrate more on the design process, and not on the development of the initial software application; this is the main aim of the project.

### Sound quality:

Since MP3 is a lossy compression technique, the quality of the compressed sound is of high importance. The technique is base on removing the superfluous sounds and reduces the size of the file, which might cause harmonic distortions. The quality of compressed sound and the ratio of Compression are directly linked and are mostly a trade-off.

However, apart from the harmonic distortion there other sound deficiencies or “artefacts” as they are called, which are dependent on the parameters and sophistication of the encoder [15]. The artefacts to look for are pre-echo, over-ring and bandwidth loss.

There are three different methods for determining the quality in the literature [15]. They are Listening test, Objective measurement techniques and Perceptual measurement techniques.  The use of any of these techniques however, is out of the scope of the project at this stage. Nonetheless, it is decided to use independent comparison results for quality determination.

### Candidate Encoders:

There are three main encoders that were considered for usage. They are LAME, BladeEnc, Fraunhoffer (FhG) and Xing.

Table 3 summaries the encoders and compare their features against some of the criteria of fitness for our purpose.

| Encoder Name | Bit-Rate encoding | Fix Point | Multiple input | Open source |
| --- | --- | --- | --- | --- |
| LAME | CBR, ABR, VBR | NO | Yes | Yes |
| BladeEnc | CBR, |  |  | Yes |
| Xing | CBR, VBR | N/A |  | No |
| FhG | CBR, VBR | N/A |  | No |

Table 3: Encoder V Criteria

As can be seen, only two of the encoders are open-source, and none are fixed point.  BladeEnc does not support VBR encoding.  LAME has more of the desirable features than any of the others.

## Independent Test Results, Methods and Test data:

There are four comprehensive comparison tests carried out by independent bodies [3,8, 9, 13]. Three of the tests are comparing MP3 encoders directly while the fourth one is comparing MP3 against CD and a different technology name OGG. Table 4 is a summary of what the tests found on the Internet.  From this data, it becomes apparent that LAME emerged on top in most of the listening tests.

| Test subject | Bit-Rate Range | Test method | Test data | Verdict | Tester |
| --- | --- | --- | --- | --- | --- |
| FhG, LAME, BladeEnc | 32-64 | Visual analysis using “sonogram” | Unknown | BladeEnc produce pre-echo FhG has over-ring | [9] |
| FhG, LAME, BladeEnc | 128 kbps (VBR) | Visual analysis using “sonogram” | Unknown | BladeEnc produce pre-echo FhG has over-ring | [9] |
| FhG, LAME, BladeEnc | 196-256- 320 | Visual analysis using “sonogram” | Unknown | LAME bits the peak performance | [9] |
| FhG, Xing, LAME 3.61, BladeEnc. | 128, 196, 256 |  | Dirty Blue |  | [8] |
| FhG, Xing, LAME3.61, BladeEnc. | 128, 196, 256 |  |  |  | [8] |
| FhG, Xing, LAME 3.61, BladeEnc. | 128, 196, 256 | Listening |  |  | [8] |
| LAME 3.89b, FhG | 128 kbps |  |  | LAME top or near top, Xing not suitable candidate | [13] |
| LAME, OGG, CD | 128 |  | Crush On You, Mano a Mano What A Feeling Ist Deine Liebe Echt Autumn Tactics | OGG is leader | [3] |
| LAME, OGG, CD | 160 |  |  | OGG is leader | [3] |
| LAME, OGG, CD | 192 |  |  | LAME better on a whole | [3] |
| LAME, OGG, CD | 256 |  |  | LAME better with high frequency OGG good with middle | [3] |
| LAME, OGG, CD | 320-350 |  |  | Both encoders do excellent | [3] |

Table 4 Test result summary

## Public domain usage:

Table 5 shows the frequency of publicly used encoders. The list is

|  | Name | Encoder included | Platform supported |
| --- | --- | --- | --- |
| 1 | AltoMP3 Maker | LAME Encoder | Win 95/98/2000/NT/ME |
| 2 | N-Koder |  | Win 95/98/NT/2000 |
| 3 | Easy CD Ripper |  | Win 98/NT/ME/2000/XP |
| 4 | YAMP |  | Win 95/98/2000/NT |
| 5 | Adv. Encode Decode Tools |  | Win 95/98/NT/2000/ME |
| 6 | MPAction Tools |  | Win 95/98/NT/2000 |
| 7 | Audio Companion |  | Win 95/98/2000/NT |
| 8 | Rob Fantastic MP3 Encoder |  | Win 95/98/2000/NT |
| 9 | Waver |  | Win 95/98/2000/NT/ME |
| 10 | Easy CD-DA Extractor |  | Win 95/98/2000/N/XP |
| 11 | Mighty DAC |  | Win 98/ME/2000/N/XP |
| 12 | CD Stripper |  | Win 95/98 NT/2000/ME |
| 13 | JukeIt Up |  | Win 95/98 2000/ME |
| 15 | CDex |  | Win 95/98/NT |
| 16 | dbPoweramp |  | Win 95/98/NT4/ME/2000 |
| 17 | FreeRip MP3 |  | Win 95/98/NT4/ME/2000 |
| 18 | CD to MP3 Maker |  | Win 95/98/NT/ME/2000 |
| 19 | Xing Encoder | Xing Encoder | Win 95/98/NT |
| 20 | Audio Catalyst |  | Win 95/98/NT |
| 21 | Froebis MP3 Studio |  | Win 95/98/NT |
| 22 | MusicBoss Grabber |  | Win 95/98 |
| 23 | MP3Enc | Fraunhoffer Encoder | Win 95/NT |
| 24 | Rosoft Audio Tools |  | Win 95/98/NT/2000 |
| 25 | AudioActive Production Studio |  | Win 95/98/NT |
| 26 | MusicMatch |  | Win 95/98/ME/NT/2000 |
| 27 | Siren Jukebox |  | Win 98/2000/ME |
| 28 | MP3HomeStudio |  | Win 95/98/ME/2000 |
| 29 | Audio Companion | BladeEnc Encoder | Win 95/98/2000/NT |
| 30 | Rob Fantastic MP3 Encoder |  | Win 95/98/2000/NT |
| 31 | Waver |  | Win 95/98/2000/NT/ME |
| 32 | CD Copy |  | Win 95/98 NT/2000 |
| 33 | MP3 Strip It! |  | Win 32/95/98 2000/NT4 |
| 34 | RightClick |  | Win 95/98/NT |
| 35 | Audio MP3 Maker |  | Win 95/98/NT/ME/2000 |
| 36 | Blaze Media Pro | Other | Win 95/98/NT/2000/ME |
| 37 | AudioWriter |  | Win 95/98/NT/2000 |
| 38 | AudioConvert |  | Win 95/98/2000/NT/ME |
| 39 | TsunamiPro |  | Win 95/98/NT |
| 40 | HyCD |  | Win 95/98 2000/NT/ME |
| 41 | MP3 to Exe |  | Win 95/98/NT |
| 42 | PC Music |  | Win 95/98 |
| 43 | SoundLimit |  | Win 95/98/NT |

Table 5 Public domain usage

As the table clearly shows, LAME is the most commonly used encoder for the Windows® environment.  Frequency of usage does not necessarily reflect quality, (for example, the VHS/Betamax scenario in the 1980s).  However, from Table 4, LAME was shown to be capable of producing very high quality audio.

## Conclusion:

Having compared several MP3 encoders, it was found that LAME has more desirable characteristics than the other encoders. It is open source, and widely used throughout the public domain, and as such it has been subject to rigorous peer appraisal.  It has a broad range of features, and a multitude of GUIs.   Although LAME does not have a fixed-point implementation, its success in all the other categories, has led to it being chosen as a base for our project.

# Cryptography

This section will contain a brief overview of encryption techniques.  After a comparison of the surveyed techniques, an encryption algorithm will be chosen.  This research has already been conducted, although not formalised, and Pretty Good Privacy (PGP) was chosen for implementation.  This will allow us to use public key RSA, along with some form of pass-phrase.

## Walkthrough

Upon initial use of the system, the user will be prompted for a pass-phrase to secure his private key.  At this point, the system will generate a public/private key pair.  The private key will be “locked” using the memorable pass-phrase, and the public key can be broadcast or placed somewhere it may be accessed by other users.

Anyone will be able to obtain and use a given public key to encrypt an MP3.  The MP3 can then be transmitted or stored securely, with very little danger of the information being read by an unwanted third-party. The encrypted MP3 can be stored on the user’s computer.  When the user comes to play MP3s, s/he will be prompted once for the pass-phrase.  This pass-phrase will “unlock” the PGP private key, which can then be used to decrypt, decode and play the MP3.

## Summary

A summary of reasons for choosing PGP over other alternatives is presented below:

- It is an open standard, which has been tried and tested over several years.

- It is so far conjectured to be cryptographically secure given a sufficient key size.

- It includes an implementation of a public key encryption algorithm (RSA).  This means data can be encrypted by anyone who has access to the user’s public key, but only decrypted by the user with the relevant private key.

- It incorporates a facility for a pass-phrase to promote ease of use.  More specifically, it is easier to remember a meaningful, user-defined pass-phrase than to remember the random string of numbers that constitute the private key.

# GUI

# System Architecture

# Processor Core Design

## Introduction

In the following section we will outline the design of the hardware, specifically the detailed design of the ASIP.  The initial aim will be to develop a working processor model running on the FPGA, capable of executing short sequences of hand-written assembly code.  From here, we will attempt to re-target the C compiler, GCC to compile programs of a higher degree of complexity for our processor.  Finally, we intend to tune the processor to the encryption and compression applications.  This will be done in the form of customising the instruction set as described in [Generating Instruction Sets and Microarchitectures from Applications]

## Hardware Overview

Figure 8: Initial Processor Model

We have decided to implement a generic RISC family core as these are well documented, and existing implementations of GCC can be converted to accommodate the customised instruction set.  Initially, we will focus on what is known as coarse grain optimisation and customisation.  In this sense, we use the word “optimisation” to mean a “tuning” of the processor to make it more specific to the target application.  This may involve removing redundant instructions or processor features, or perhaps improving existing features.  Rather than just concentrating on getting the fastest computation times out of the processor, we also aim to use FPGA resources more efficiently than a general-purpose processor would.  For example, if the compiler only references four registers in the generated byte-code, only four registers need to be implemented on the FPGA.

If time permits, it is hoped that once the coarse grain optimisations have been completed we will evolve the architecture to Very Long Instruction Word (VLIW). Using VLIW architecture will enable a higher degree of parallelism to be exploited on the FPGA, by executing certain instructions (that would otherwise be serial) in parallel.

### Why RISC?

A RISC processor model was chosen due to popular conjecture that approximately 80% of the code is executed using 20% of the instructions.  Because more complex instructions take longer to execute, RISC processors have a significant performance advantage over CISC processors.  They are much more efficiently pipelined, and can therefore exploit a higher degree of parallelism.

By selectively refining the most commonly used instructions, we can aim also to optimise these, and then synthesise any further operations from these initial ones. Space considerations were also a factor in deciding upon a suitable instruction set.  Although CISC architecture substantially reduces the overall program size, it is important to note that RISC significantly decreases the number of gates required on the FPGA.

### Harvard Architecture

The core will implement a Harvard architecture, which uses separate data and program buses.   This architecture is less common than Von Neumann architecture, due to the additional number of pins required when implementing the processor in real hardware.  This is not a consideration in the FPGA implementation, as the number of pins is fixed.  The perceived main advantage to implementing Harvard architecture is the performance gain achieved by separating the interests of data and processor instructions.  This allows both data and instructions to be written and read simultaneously, with neither being restricted by a shared bus.  We concede that Harvard is more difficult to implement than Von Neumann architecture, so as part of our risk management strategy we intend to prototype our system with Von Neumann.  As we get more confident with the development environment, we will then migrate to Harvard.

### Memory Banks

The FPGA has four memory banks, each of two megabytes.  We believe that one bank should be sufficient to store the program itself.  This allows us three banks for data – one for a heap, one for a stack and one for general application data.

### Registers

The registers will initially be 32-bits wide, as this makes targeting of GCC slightly less complex.  At a later stage, these may be decreased if it is believed that there is a performance advantage in doing this (either in time or space complexity).  There will probably be at least 16 registers, most of which will be general purpose.  There will also be a program counter, and an X-Register to store intermediate results from the ALU. This is necessary as initial observations suggest there are a significant number of multiply and accumulate operations in MP3 encoding and encryption.  Furthermore, a status register will store state values for the ALU, indicating such conditions as overflow, underflow, carry, etc.

## MIPS

The MIPS (Microprocessor without Interlocked Piped Stages) has been frequently used as a teaching model for students. This is centred on the MIPS simple instruction set and its logical load store architecture.

We chose to implement a MIPS instruction set for the following reasons. Firstly it a RISC core processor. This means it uses load-store architecture with three-register operations. This provides an initial simpler approach to addressing when compared with other models. Furthermore register operations have a lower latency than equivalent in-memory operations, and makes simpler for compilers to optimise register allocation. An additional reason for choosing the MIPS as a starting point is it is open source with several versions available for download off the Internet. Furthermore GCC has already been targeted to compile for MIPS based architecture. This provides a solid platform from which we can further develop the instruction set. The final reason we chose a MIPS as a starting point was prior experience.

## Development Process

Figure 9: ASIP Development Process

### C Source Code

The C source code will be an application, which has been designed and written to compile and execute on a generic PC architecture.  In the case of our system, this will consist of an MP3 encoder/decoder and a simple public key encryption algorithm.  It is essential that the source code is fully functional on the PC and will compile successfully using the Gnu C Compiler, GCC, without error.

### Profiling

Profiling will essentially determine which parts of the compiled code are “hotspots”, i.e. where a significant proportion of the execution time is focussed.  As this is a complex process, a discussion of the intricacies and methods that will be invoked is contained elsewhere in this document.

### Static Analysis

Static Analysis is the process of obtaining a breakdown of the instruction usage for a piece of source code, simply by counting the different instructions for a specific piece of application assembly language.  One or more sections of code will be selected to undergo this analysis, from the “hotspots” identified by the profiling process.  This code will then be compiled (but not assembled) for the generic MIPS architecture.  The aim here is to decide which instructions are most and least frequently used, so that sections of the MIPS processor implementation we aim to use as a starting point may be removed if deemed redundant.  Static analysis will also give an indication of register usage, which can be used to fine-tune the processor model.

### GCC

The output produced by GCC will be a three-address format based on RISC-like load-store architecture.  In this model, only the load and store operations are able to operate on main memory, with all other instructions operating on registers.  GCC already has an implementation to produce MIPS code, which we will use as a starting point for the ASIP.  This version also includes performance optimisations for the MIPS processor.

### Simulation

The aim of simulation is to ensure the version of the software compiled and run on the ASIP is functionally equivalent to the original software implementation.  The outputs of the two programs are compared to identify differences that may signify errors in either the compiler or the processor model.  Due to architecture differences, e.g. floating point vs. fixed-point arithmetic, reasonable tolerances and methods of comparison must be identified.  This may include subjective metrics such as listening tests as well as more rigidly defined comparisons.  As we are not expert listeners, we will also use a direct comparison with some statistical analysis of the data to determine the average deviation.

### Further Instruction Set Analysis

Initial analysis will focus on minimising the number of instructions performed in total by the processor for common instruction sequences.  It is envisaged that the development cycle of the processor model will terminate when one of two outcomes occurs.  Either the maximum number of gates on the FPGA will be reached, or the processor will have developed such that any modifications to the instruction set will have a detrimental effect on the execution speed.  This will be determined with rigorous benchmarking.

### Changes & Optimisations

Changes that will be made to the processor model will take two forms.

- Mandatory changes signified by incorrect results being produced from simulation.  These are likely to be caused by errors in the conversion of GCC for the processor model, or by errors in the Handel-C implementing the processor model.  These changes are necessary for the successful operation of the system.

- Optimisation changes identified during analysis of the code.  These changes are likely to be in the form of removing redundant instructions, or perhaps making minor adjustments to the processor model itself in order to improve efficiency and resource usage.  They also may include combining simple operations into Very Long Instruction Word (VLIW) operations to exploit a higher degree of parallelism.

Changes and optimisations will occur during each iteration of the spiral model.  This will result a processor model that will evolve rather than the end artefact being initially fully designed.

References:

- Mp3 compression,  Andrei Gule, http://www.digit-life.com/articles/mp3comp/index.html, ( sited:  11/11/2001)

- Research quality of coding sound by different mp3 encoders, Aleksey Lukyanov http://www.digit-life.com/articles/mp3quality/index.html (sited: 11/11/2001 )

- OGG vs. LAME, Andrei Aspidov,  http://www.digit-life.com/articles/oggvslame/index.html, (sited: 11/11/2001)

- Encoder Quality Comparison 1.8.2, December 16, 1999. http://www.raum.com/mpeg/reviews_quality.html

- Which is the best low-bitrate audio compression algorithm?, Panos Stokas sited @ http://ekei.com/audio/ (sited: 11/11/2001)

- BladeEnc,  Tord Jansson, http://bladeenc.mp3.no/(16/11/2001)

- http://www.iis.fhg.de/amm/download/mp3encman/index3.html

- Do MP3 encoders sound different? by Will Ryu, http://arstechnica.com/wankerdesk/1q00/mp3/mp3-2.html  (sinted: 19/11/2001)

- Encoder Analysis and Visualisation, Chris Jonson, http://www.airwindows.com/encoders/index.html (sidted: 19/11/2001)

- http://www.xingtech.com/(sidted 19/11/2001)

- http://www.sulaco.org/mp3/(sidted :19/11/2001)

- http://www.r3mix.net/ (sited: 19/11/2001)

- Discussion of Audio Compression, http://fastforward.iwarp.com/ (sited: 19:11:2001)

- http://www.mp3-converter.com/mp3_converter_full.htm (sited: 19//11/2001)

- An Introduction to MPEG Layer-3,  K. Brandenburg, and H. Popp, http://www.mp3-tech.org/

- http://www.mp3-tech.org/ (sited: 11/11/2001)

- http://www.mp3dev.org/mp3/doc/html/modes.html

# Appendix

Here is a presentation of the frame header content. Characters A to M are used to indicate different fields. In the table below, you can see details about the content of each field.

AAAAAAAA AAABBCCD EEEEFFGH IIJJKLMM

| Sign | Length (bits) | Position (bits) | Description |
| --- | --- | --- | --- |
| A | 11 | (31-21) | Frame sync (all bits must be set) |
| B | 2 | (20,19) | MPEG Audio version ID 00 - MPEG Version 2.5 (later extension of MPEG 2) 01 – reserved 10 - MPEG Version 2 (ISO/IEC 13818-3) 11 - MPEG Version 1 (ISO/IEC 11172-3) |
| C | 2 | (18,17) | Layer description 00 - reserved 01 - Layer III 10 - Layer II 11 - Layer I |
| D | 1 | (16) | Protection bit 0 - Protected by CRC (16bit CRC follows header) 1 - Not protected |
| E | 4 | (15,12) | Bitrate index bits; V1,L1; V1,L2; V1,L3; V2,L1; V2, L2 & L3. 0000; free; free; free; free; free. 0001; 32; 32; 32; 32; 8. 0010; 64; 48; 40; 48; 16. 0011; 96; 56; 48; 56; 24. 0100; 128; 64; 56; 64; 32. 0101; 160; 80; 64; 80; 40. 0110; 192; 96; 80; 96; 48. 0111; 224; 112; 96; 112; 56. 1000; 256; 128; 112; 128; 64. 1001; 288; 160; 128; 144; 80. 1010; 320; 192; 160; 160; 96. 1011; 352; 224; 192; 176; 112. 1100; 384; 256; 224; 192; 128. 1101; 416; 320; 256; 224; 144. 1110; 448; 384; 320; 256; 160. 1111; bad; bad; bad; bad; bad. NOTES: All values are in kbps V1 - MPEG Version 1 V2 - MPEG Version 2 and Version 2.5 L1 - Layer I L2 - Layer II L3 - Layer III "free" means free format. The free bit rate must remain constant, and must be lower than the maximum allowed bit rate. Decoders are not required to support decoding of free bit rate streams. "bad" means that the value is un-allowed. MPEG files may feature variable bit rate (VBR). Each frame may then be created with a different bit rate. It may be used in all layers. Layer III decoders must support this method. Layer I & II decoders may support it. |
| F | 2 | (11,10) | Sampling rate frequency index bits; MPEG1; MPEG2; MPEG2.5. 00; 44100 Hz; 22050 Hz; 11025 Hz. 01; 48000 Hz; 24000 Hz; 12000 Hz. 10; 32000 Hz; 16000 Hz; 8000 Hz. 11; reserv.; reserv.; reserv.. |
| G | 1 | (9) | Padding bit 0 - frame is not padded 1 - frame is padded with one extra slot Padding is used to exactly fit the bitrate.As an example: 128kbps 44.1kHz layer II uses a lot of 418 bytes and some of 417 bytes long frames to get the exact 128k bitrate. For Layer I slot is 32 bits long, for Layer II and Layer III slot is 8 bits long. |
| H | 1 | (8) | Private bit. This one is only informative. |
| I | 2 | (7,6) | Channel Mode 00 - Stereo 01 - Joint stereo (Stereo) 10 - Dual channel (2 mono channels) 11 - Single channel (Mono) Note: Dual channel files are made of two independant mono channel. Each one uses exactly half the bitrate of the file. Most decoders output them as stereo, but it might not always be the case. One example of use would be some speech in two different languages carried in the same bitstream, and then an appropriate decoder would decode only the choosen language. |
| J | 2 | (5,4) | Mode extension (Only used in Joint stereo) Mode extension is used to join information that is of no use for stereo effect, thus reducing needed bits. These bits are dynamically determined by an encoder in Joint stereo mode, and Joint Stereo can be changed from one frame to another, or even switched on or off. Complete frequency range of MPEG file is divided in subbands There are 32 subbands. For Layer I & II these two bits determine frequency range (bands) where intensity stereo is applied. For Layer III these two bits determine which type of joint stereo is used (intensity stereo or m/s stereo). Frequency range is determined within decompression algorithm. Layer I and II; Layer III. value; Layer I & II. 00; bands 4 to 31. 01; bands 8 to 31. 10; bands 12 to 31. 11; bands 16 to 31.; Intensity stereo; MS stereo. off; off. on; off. off; on. on; on.. |
| K | 1 | (3) | Copyright 0 - Audio is not copyrighted 1 - Audio is copyrighted The copyright has the same meaning as the copyright bit on CDs and DAT tapes, i.e. telling that it is illegal to copy the contents if the bit is set. |
| L | 1 | (2) | Original 0 - Copy of original media 1 - Original media The original bit indicates, if it is set, that the frame is located on its original media. |
| M | 2 | (1,0) | Emphasis 00 - none 01 - 50/15 ms 10 - reserved 11 - CCIT J.17 The emphasis indication is here to tell the decoder that the file must be de-emphasized, i.e. the decoder must 're-equalize' the sound after a Dolby-like noise suppression. It is rarely used. |

http://www.iis.fhg.de/amm/techinf/layer3/index.html

Davis Pan, A Tutorial on MPEG/ Audio Compression, IEEE Multimedia Journal, Summer 1995

K. Brandenburg, H. Popp, An Introduction to MPEG Layer-3, Fraunhofer Institut fur Integrierte Schaltungen (IIS)

MPEG Audio Layer I/II/III frame header, http://www.mp3-tech.org/programmer/frame_header.html,

http://www.mp3-converter.com/mp3codec/mp3_anatomy.htm

www.id3.org/mp3frame.html

Guy Hart-Davis, Rhonda Holmes, MP3: I didn’t know you could do that, SYBEX Inc, 1999

http://www.dictionary.com/cgi-bin/dict.pl?term=stereo

http://csunix1.lvc.edu/~snyder/ovrsamplng2.html

J.O. Smith and P. Gossett, "A Flexible Sampling-Rate Conversion Method," Proc. IEEE ICASSP, vol. 2 , pp. 19.4.1-19.4.2, San Diego, March 1984.

Single Supply Dual 16-bit Audio DAC, AD1866 Data Sheet, Analog Devices, Rev. 0.

Generating Instruction Sets and Microarchitectures from Applications, Ing-Jer Huang and Alvin M. Despain, Dept of Electrical Engineering – Systems, University of Southern California (Date unknown).

MIPS32 Architecture for Programmers volume I: Introduction to the MIPS32 Architecture, MIPS Technologies Inc. (March 2001).